Assessment Brief

| Module Code | UFCF93-30-1 |
| --- | --- |
| Assessment Name | Lab Book Wireshark lab: Ethernet and ARP |
| Assessment Type | Regular |
| Module Name | Computer and Network Systems |
| Year | 2025 |
| Weighting | 12.5% of overall weightage |
| Duration | 6 weeks |
| Learning outcomes tested | MO3 |

Reference : The tasks are designed based on the following reference

Website (wireshark lab): Jim Kurose Homepage (umass.edu)

Book : Kurose, James F. Ross, Keith W., 2021. Computer   Networking: A Top-Down Approach, Pearson Education

# Wireshark Lab: Ethernet and ARP

In this lab, we’ll investigate the Ethernet protocol and the ARP protocol.  Before beginning this lab, you’ll probably want to review sections 6.4.1 (Link-layer addressing and ARP) and 6.4.2 (Ethernet) in the text. RFC 826 (ftp://ftp.rfc-editor.org/in-notes/std/std37.txt) contains the gory details of the ARP protocol, which is used by an IP device to determine the IP address of a remote interface whose Ethernet address is known.

Capturing and analyzing Ethernet frames

The Let’s begin by capturing a set of Ethernet frames to study.  To do this, of course, you’ll need access to a wired Ethernet connection for your PC or Mac – not necessarily a common scenario these days, given the popularity of wireless WiFi and cellular access. If you’re unable to run Wireshark on a live Ethernet connection, you can download ethernet-wireshark-trace file from blackboard.  In addition, you may well find it valuable to download this trace even if you’ve captured your own trace and use it, as well as your own trace, when you explore the questions below.

Do the following:

- First, make sure your browser’s cache of previously downloaded documents is empty.

- Start up Wireshark and enter the following URL into your browser: http://gaia.cs.umass.edu/wireshark-labs/HTTP-wireshark-file3.html. Your browser should display the rather lengthy US Bill of Rights.

- Stop Wireshark packet capture.

First, find the packet number (the leftmost column in the upper Wireshark window) of the HTTP GET message that was sent from your computer to gaia.cs.umass.edu, as well as the beginning of the HTTP response message sent to your computer by gaia.cs.umass.edu.  You should see a screen that looks something like this (where packet 126 in the screen shot below contains the HTTP GET message)

Figure 1: Wireshark display showing HTTP GET message for http://gaia.cs.umass.edu/wireshark-labs/HTTP-wireshark-file3.html

Since this lab is about Ethernet and ARP, we’re not interested in high-level protocols like IP, TCP or HTTP.  We’re interested in Ethernet frames and ARP messages!

Let’s start by looking at the Ethernet frame containing the HTTP GET message. (Recall that the HTTP GET message is carried inside of a TCP segment, which is carried inside of an IP datagram, which is carried inside of an Ethernet frame; reread section 1.5.2 in the text if you find this notion of encapsulation a bit confusing). Expand the Ethernet II information in the packet details window.  Note that the contents of the Ethernet frame (header as well as payload) are displayed in the packet contents window.  Your display should look similar to that shown in Figure 2.

Figure 2: Wireshark display showing details of the Ethernet frame containing the HTTP GET request.

Answer following questions in blackboard (Questions: Ethernet and ARP-lab )using ethernet-wireshark-trace .

Questions

- What is the 48-bit Ethernet address of the host computer? [Marks: 5]

- What is the 48-bit destination address in the Ethernet frame?  Is this the Ethernet address of gaia.cs.umass.edu? (Hint: the answer is no).  What device has this as its Ethernet address? [Note: this is an important question, and one that students sometimes get wrong.  Re-read pages 483-484 in the text and make sure you understand the answer here.] [Marks: 5]

- What is the hexadecimal value for the two-byte Frame type field in the Ethernet frame carrying the HTTP GET request?  What upper layer protocol does this correspond to? [Marks: 5]

Next, answer the following questions, based on the contents of the Ethernet frame containing the first byte of the HTTP response message.

- What is the value of the Ethernet source address?  Is this the address of  the host computer, or of gaia.cs.umass.edu (Hint: the answer is no).  What device has this as its Ethernet address? [Marks: 5]

- What is the destination address in the Ethernet frame?  Is this the Ethernet address of your computer? [Marks: 5]

- Give the hexadecimal value for the two-byte Frame type field. What upper layer protocol does this correspond to? [Marks: 5]

- How many Ethernet frames (each containing an IP datagram, each containing a TCP segment) carry data that is part of the complete HTTP “OK 200 ...” reply message. ( hint: check number of  TCP segements ) [Marks: 5]

The Address Resolution Protocol

In this section, we’ll observe the ARP protocol in action.  We strongly recommend that you re-read section 6.4.1 in the text before proceeding.

ARP Caching

Recall that the ARP protocol typically maintains a cache of IP-to-Ethernet address translation pairs on your computer.  The arp command (in both DOS, MacOS and Linux) is used to view and manipulate the contents of this cache. Since the arp command and the ARP protocol have the same name, it’s understandably easy to confuse them. But keep in mind that they are different - the arp command is used to view and manipulate the ARP cache contents, while the ARP protocol defines the format and meaning of the messages sent and received, and defines the actions taken on ARP message transmission and receipt.

Let’s take a look at the contents of the ARP cache on your computer.  In DOS, MacOS, and Linux, the “arp -a” command will display the contents of the ARP cache on your computer.  So at a command line, type “arp -a”.  The results of entering this command on one of the authors’ computers is shown in Figure 3.

Figure 3: Executing the “arp -a” command

Answer following questions in blackboard (Questions: Ethernet and ARP-lab ) using Figure 3.

Questions

- How many entries are stored in your ARP cache? [Marks: 5]

- What is contained in each displayed entry of the ARP cache?  [Marks: 5]

In order to observe your computer sending and receiving ARP messages, we’ll need to clear the ARP cache, since otherwise your computer is likely to find a needed IP-Ethernet address translation pair in its cache and consequently not need to send out an ARP message.  The “arp –d -a” command will clear your ARP cache using the command line. In order to run this command on a Mac or Linux machine you’ll need root privileges or use sudo. If you don’t have root privileges and can’t run Wireshark on a Windows machine, you can skip the trace collection part of this lab and use the trace file ethernet-wireshark-trace discussed earlier.

Observing ARP in action

For live caputer do the following:

- Clear your ARP cache, as described above and make sure your browser’s cache is cleared of previously downloaded documents.

- Start up the Wireshark packet sniffer.

- Enter the following URL into your browser:
http://gaia.cs.umass.edu/wireshark-labs/HTTP-wireshark-lab-file3.html
Your browser should again display the rather lengthy US Bill of Rights.

- Stop Wireshark packet capture.

Again, we’re not interested in IP or higher-layer protocols, so let’s just look at ARP packets.  Your display should look similar to that shown in Figure 3 (note we have entered “arp” into the display filter window at the top of the Wireshark screen).

Figure 3: An ARP query being broadcast from the computer of one of the authors’

Let’s start by looking at the Ethernet frames containing ARP messages.

Answer following questions in blackboard (Questions: Ethernet and ARP-lab )using ethernet-wireshark-trace .

Questions

- What is the hexadecimal value of the source address in the Ethernet frame containing the ARP request message sent out by the host computer? ( hint: you ned to look at packet number 108 in wireshark) [Marks: 5]

- What is the hexadecimal value of the destination addresses in the Ethernet frame containing the ARP request message sent out by the host computer? And what device(if any) corresponds to that address (e.g,, client, server, router, switch or otherwise...)? [Marks: 5]

- What is the hexadecimal value for the two-byte Ethernet Frame type field.  What upper layer protocol does this correspond to? [Marks: 5]

Now let’s dig even a bit deeper into the ARP messages themselves.  To answer this question, you will need to dig into ARP.  The original RFC (https://datatracker.ietf.org/doc/html/rfc826) that defines ARP is a little hard to read. The Wikipedia entry for ARP is pretty good: https://en.wikipedia.org/wiki/Address_Resolution_Protocol

Answer the following question about the ARP request message sent by your computer.

- How many bytes from the very beginning of the Ethernet frame does the ARP opcode field begin?  ( hint: you need to count from start of Ethernet to opcode field in ARP) [Marks: 5]

- What is the value of the opcode field within the ARP request message sent by your computer? [Marks: 5]

- Does the ARP request message contain the IP address of the sender?  If the answer is yes, what is that value? [Marks: 5]

- What is the IP address of the device whose corresponding Ethernet address is being requested in the ARP request message sent by your computer? [Marks: 5]

Now find the ARP reply message that was sent in response to the ARP request from the host computer.

- What is the value of the opcode field within the ARP reply message received by your computer? [Marks: 5]

- Finally (!), let’s look at the answer to the ARP request message! What is the Ethernet address corresponding to the IP address that was specified in the ARP request message sent by the host computer (see question 18)? [Marks: 5]

We’ve looked the ARP request message sent by your computer running Wireshark, and the ARP reply sent in reply.  But there are other devices in this network that are also sending ARP requests that you can find in the trace.

- We’ve looked the ARP request message sent by your computer running Wireshark, and the ARP reply message sent in response.  But there are other devices in this network that are also sending ARP request messages that you can find in the trace. Why are there no ARP replies in your trace that are sent in response to these other ARP request messages?  [Marks: 5]

Extra Credit  [Marks: 2.5 + 2.5]

EX-1. The arp command:

arp -s InetAddr EtherAddr

allows you to manually add an entry to the ARP cache that resolves the IP address InetAddr to the physical address EtherAddr. What would happen if, when you manually added an entry, you entered the correct IP address, but the wrong Ethernet address for that remote interface? A security attack known as “ARP poisoning” https://www.varonis.com/blog/arp-poisoning/ spoofs ARP messages and causes incorrect entries to be made into an ARP table!

EX-2.  What is the default amount of time that an entry remains in your ARP cache before being removed?  You can determine this empirically (by monitoring the cache contents) or by looking this up in your operating system documentation.  Indicate how/where you determined this value.

References to figures and sections are for the 8th edition of our text, Computer Networks, A Top-down Approach, 8th ed., J.F. Kurose and K.W. Ross, Addison-Wesley/Pearson, 2020.